IIIT-B Chip Design Studio Weekly Report

Week: 2

Team Name/Project: Team Friday - RISC V Single Cycle CPU

Team Members:

Namai Chandra 23f3000200@es.study.iitm.ac.in

Madhur Thareja 23f3000178@es.study.iitm.ac.in

Aditya Prakash 23f3000345@es.study.iitm.ac.in

Vivek Dhariwal 23f3000338@es.study.iitm.ac.in

# 1. Updates

Current Progress:

After successfully gaining hands-on experience with the software and synthesizing the RISC-V single-cycle CPU in the previous week, the Graphic Database System (GDS) of the CPU from the has now been generated. Additionally, we have synthesized and elaborated our pipelined CPU. Having met our objectives, we are now focused on exceeding expectations and introducing novelty in the remaining two weeks. To achieve this, we are moving forward with the PYNQ-Z2 board.

Challenges Faced:

- The initial attempts to synthesize the pipelined CPU encountered issues due to multiple black boxes caused by incorrect module instantiations. While the CPU successfully synthesized in Genus when the ‘genus_script’ was executed, running ‘check_design’ clearly indicated the presence of unresolved black boxes.

- Despite repeated attempts, the port outputs did not reflect correctly during the RTL to GDS process. This issue required resynthesizing both the pipelined and single-cycle CPUs before systematically debugging and correcting each block individually to ensure proper functionality.

- The input and output definitions in the pipelined CPU were not properly configured, as the original code was initially designed for FPGA implementation and was not inherently synthesizable for on-chip applications.

- Setting up the PYNQ board and getting acquainted with its environment was time-consuming, as it required resynthesizing the files from Quartus Prime to Xilinx Vivado, rather than using Cadence Genus. This transition was necessary to ensure compatibility with the FPGA architecture on the PYNQ-Z2.

- GitHub Link: Provide the GitHub repository link for the project, if any:

Repository: https://github.com/NamaiBest/iiitb_chip_design_studio

- Project Idea

In addition to what was mentioned in the week 1 projct idea report, this is a continuation of that...

The pipelined RISC-V CPU is significantly more capable of handling TinyML tasks, and once hardware-accelerated, its performance will be greatly enhanced. In parallel, when powered using low-power ICs and ambient energy sources, its potential applications become vast. With the right TinyML algorithm, this concept could even be explored for brain-implanted devices, utilizing electrical impulses from neural activity as a power source. The CPU would then autonomously decode neural signals, paving the way for a fully self-sustaining neural implant but that's certainly for a future application.

Following the successful synthesis of the pipelined RISC-V CPU, the next phase involves implementing MicroSpeechon the PYNQ-Z2. This decision is driven by the onboard audio jack, which enables efficient real-time voice processing. Unlike traditional FPGAs such as the DU Nano, the PYNQ-Z2 integrates an FPGA fabric with an ARM Cortex-A9 processor, allowing for seamless interaction between hardware and software through Python.

To ensure compatibility with the PYNQ-Z2 architecture, the CPU is required to be migrated from Quartus to Xilinx Vivado. This process includes resynthesizing the design, integrating AXI interfaces, and generating the necessary bitstream (.bit) and hardware description file (.hwh). Once deployed, Python will facilitate dynamic interaction with the FPGA logic, enabling real-time execution of TinyML models. This approach leverages hardware acceleration alongside Python-based control, enhancing both performance and flexibility for machine learning applications.

# 4. Schematic/Simulations

Include schematics, simulation results, or any relevant visuals:

- Schematics & Simulation Results:

RTL TO GDS Single Cycle CPU

Clock Tree

Terminal Success!

5. Analysis:

1. Synthesis & Placement

- Total instances placed: 5,373

- Movable instances: 5,373

- Placement density: 77.69% (21,856/28,132 sites used)

- Maximum instance displacement: 14.93 µm

- Hierarchical instances deleted: 16

- Sequential instances deleted: 1,024

2. Power & Routing

- Total vias generated: 45,449

- Single-cut vias: 45,448 (99.99%)

- Multi-cut vias: 1 (0.01%)

- Total wire length: 102,627 µm

- Total half-perimeter of net bounding box: 111,057 µm

- Metal layers used: 6 (Metal1 to Metal6)

- Routing overflow: 0.00% (H & V)

3. Clock Tree & Timing

- Total clock nets: 171

- Clock instances placed: 156

- Clock buffer distribution: CLKBUFX4: 143, CLKBUFX3: 13

- Clock wire length: 7,869 µm

- Total capacitance: 1.149 pF

- Worst Negative Slack (WNS): 5.978 ns

- Total Negative Slack (TNS): 0.000 ns

4. Area Utilization

- Total area before optimization: 21.86 billion µm²

- Total area after optimization: 18.40 billion µm²

- Area reduction: 15.8%

- Optimized datapath elements: 10

5. Optimization & Design Rule Violations (DRV)

- Clock tree optimization applied: Yes

- Total design runtime: 4 hours, 16 minutes

- Maximum move in detailed placement: 14.82 µm

- Clock density: 77.49% after optimization

Key Findings

Similar to an issue we encountered last week, our pipelined CPU synthesized successfully in Quartus, but in Cadence Genus, it generated black boxes. This discrepancy arises because Quartus synthesizes designs for FPGAs, which are reprogrammable, whereas Genus follows the ASIC flow, which requires fixed, fully synthesizable logic. Unlike FPGAs, ASICs are hardwired for specific applications, offering higher performance, lower power consumption, and optimized area efficiency. The issue was resolved by making the design fully synthesizable, primarily by converting initial blocks into always blocks and implementing a global reset, ensuring that conditions execute only once while preserving the intended initialization logic.

Our initial implementation using a single-cycle RISC-V CPU was functionally correct but proved insufficient for executing ML workloads efficiently. The single-cycle architecture processed each instruction in one clock cycle, leading to long execution times for complex operations like matrix multiplications and convolutions, which are critical for TinyML, TensorFlow Lite Micro, and MicroSpeech. Additionally, the lack of instruction overlap resulted in poor resource utilization and lower throughput. To overcome these limitations, we transitioned to a pipelined RISC-V CPU, which allows for parallel instruction execution, higher throughput, and better performance for ML applications, making it a more suitable choice for deployment on the PYNQ-Z2 board.

Insights or Learnings

The transition from FPGA-based design to ASIC implementation required significant modifications, particularly in port definitions, synthesis constraints, and design optimization. Key takeaways include the importance of defining proper I/O constraints, ensuring synthesizability across different tools (Quartus, Vivado, Genus), and addressing black-box issues caused by incorrect module instantiations. The RTL to GDS flow highlighted challenges in timing constraints, power routing, and placement density, emphasizing the need for precise clock tree synthesis and resource optimization. Additionally, integrating TinyML on the PYNQ-Z2 reinforced the advantages of hardware acceleration for ML workloads, requiring efficient AXI-based communication between the FPGA fabric and the ARM processor.

Key learnings -

- FPGA to ASIC Transition – Code designed for FPGA requires modifications for ASIC synthesis, particularly in port definitions, reset logic, and module instantiations.

- Synthesis Compatibility – Differences in synthesis tools (Quartus, Vivado, Genus) require restructuring HDL code and ensuring proper timing constraints.

- Power & Routing Challenges – Proper VDD/VSS connections, power stripe generation, and clock tree synthesis are critical for successful GDS implementation.

- Timing-Driven Placement – Defining constraints early helps prevent timing violations and improves resource utilization.

Improvements or Modifications Needed

Our current focus is on implementing the RISC-V pipelined CPU on the PYNQ-Z2 board, which features a Xilinx Zynq-7000 SoC with a dual-core ARM Cortex-A9 processor and programmable FPGA fabric. Since our initial implementation was done using Intel Quartus Prime, we need to adapt our design to be compatible with Xilinx toolchains. This involves:

- Converting Code for Xilinx Vivado – Modifying our HDL code to ensure compatibility with Xilinx synthesis tools.

- Generating Synthesizable RTL – Refining our design to be fully synthesizable for the FPGA fabric of the PYNQ-Z2.

- Optimizing for FPGA Constraints – Adjusting timing, resource utilization, and power constraints for efficient execution on the Zynq-7000 SoC.

- Integrating AXI Interfaces – Ensuring seamless communication between the programmable logic and the ARM processing system using AXI interconnects.

Beyond this, we also plan to leverage the onboard audio jack of the PYNQ-Z2 to implement MicroSpeech, enabling basic voice recognition. Additionally, we may use TensorFlow Lite Micro, as the PYNQ-Z2 supports Python programming, making it easier for us to implement our ML models. This will serve as an entry point for deploying TinyML, allowing us to run lightweight machine learning models directly on the board.

These enhancements will not only validate our CPU implementation but also expand its capabilities into embedded AI applications.