IIIT-B Chip Design Studio Weekly Report

Week: 3

Team Name/Project: Team Friday - RISC V Single Cycle CPU

Team Members:

Namai Chandra 23f3000200@es.study.iitm.ac.in

Madhur Thareja 23f3000178@es.study.iitm.ac.in

Aditya Prakash 23f3000345@es.study.iitm.ac.in

Vivek Dhariwal 23f3000338@es.study.iitm.ac.in

# 1. Updates

Current Progress:

The design is in the final stages of implementation, following the successful synthesis of the single-cycle RISC-V CPU. After synthesis, the design underwent implementation and bitstream generation. The CPU has now been successfully deployed on the De0 board after being cross-verified and synthesized twice using Quartus Prime and Vivado. The final GDS file of the CPU includes a path-planning instruction set, designed with practical use cases in mind, further details are provided in the ‘Project Idea’ section.

Challenges Faced:

- Despite the Verilog hardware description and logic being correct, the CPU could not be implemented because the output ports exceeded the available I/O pins on the PYNQ board

- In Vivado, we initially placed all the modules of our CPU, such as the controller and datapath, separately. However, these modules are already referenced within ‘riscv_cpu.v’, making separate placement unnecessary.

- The initial GDS file, in its ‘inst_mem’, contained a ‘$readmem’ statement that attempted to fetch instructions from the ‘program.hex’ file, which holds 32-bit instructions. However, relying on this approach could not yield the correct result as ‘$readmem’ is not supported in the GDS implementation. If the instructions are not hardwired, a new module must be created to function as BRAM, allowing external instruction loading and execution.

- The ‘Run Automated Block Connection’ did not work correctly since it created unnecessary external outputs. Additionally, the testbench had to be properly imported, including the .dump file.

- An open source alternative of introducing the BRAM using an open-source tool called ‘openRAM’ was tried but it did not produce the desired output

- GitHub Link: Provide the GitHub repository link for the project, if any:

Repository: https://github.com/NamaiBest/iiitb_chip_design_studio

- Project Idea

In addition to what was mentioned in the week 1&2 project idea report, this is a continuation of that...

The Verilog modules have been exported from Quartus Prime to Vivado in order to initially implement the RISC-V single-cycle CPU on the PYNQ board. The process involved importing the code, creating the block diagram, simulating it, synthesizing it, and finally implementing it. Lastly, the bitstream was generated before being uploaded to the PYNQ board.

The next step is to run a path-planning algorithm written in C, which determines the shortest path between two points. The map is already represented as a graph in the C program, allowing efficient route calculations.

The C code processes a map with nodes, taking the current and target nodes as input. It assigns costs to adjacent nodes and determines the path with the lowest cumulative cost, guiding the bot along the most efficient route.

Practical Application -

1. Rescue Ambulance Navigation-

The intended application of this project is for offline navigation of ambulances in remote locations that lack proper mapping. The system takes two inputs:

1. Starting Node – The location where the ambulance is deployed.

2. Target Node – The location from which the emergency call is made.

Using the CPU implemented on the FPGA, the algorithm computes the optimal route, ensuring faster response times for emergency services in unmapped or poorly mapped regions. All this can be done for a fraction of the computing power and cost since the processing can be done locally on the device without the need of servers.

2. Space Robotics & Lunar/Martian Rovers

The satellite captures images of the lunar or Martian surface to generate a terrain map. Key points of interest or data collection sites are designated as nodes, which are then stored in the rover’s memory. Using this pre-mapped data, the rover can navigate autonomously without relying on GPS, which is unavailable in these environments. This method also significantly reduces power consumption compared to continuous real-time scanning and communication-based navigation.

As of the time of this report, the Verilog module has been successfully tested on hardware, including an IR array module, which accurately detects nodes and functions as intended.

Parallelly, the PYNQ board has been configured to run MicroSpeech by installing all necessary TensorFlow Lite libraries, required dependencies, and the PYNQ OS on Jupyter. Instead of using the onboard PYNQ OS for processing, we aim to have our custom CPU handle all computations. This is the primary focus of our current work. A basic form of Convolutional Neural Network (CNN) is also being worked on involving matrix multiplication efficiently

# 4. Schematic/Simulations

Include schematics, simulation results, or any relevant visuals:

- Schematics & Simulation Results:

Final RISC V GDS

Elaborated Schematic

Terminal DRC (0 Errors)

LEC Verified (Unmapped Points: None)

Schematic (Quartus Prime)

5. Analysis:

Instruction Size: 32-bit instructions

Word Size: 4 bits

Size of Instruction Memory: 512 bits /4 = 128

Reference Map:

Test Statement:

Present Node = 8

Final Node = 23

Real Time Simulation Output

For the shortest path from 8 to 11, it will indeed encounter the nodes 6,0,1 then 11. This is done through the signal tap logic analyzer present in Quartus Prime.

The pathplanner.c is running successfully, the input of start point in the first 2 location of data memory and it calculates and stores the correct path in the datamem.

Key Findings

During the implementation of the RISC-V single-cycle CPU on the PYNQ board, several challenges were encountered. Despite the Verilog hardware description and logic being correct, the CPU failed to implement due to the number of output ports exceeding the available I/O pins on the board. To address this, we attempted to use Virtual Input/Output (VIO) in Vivado to simulate inputs and outputs without requiring physical connections. Additionally, issues related to proper clocking, signal routing, and resource constraints were analyzed and resolved. After initially implementing the design in Quartus, signal tapping was performed to monitor internal signals and verify correct execution. The debugging process also involved verifying the correctness of synthesized logic through simulation before final deployment.

Insights or Learnings

Implementing the single-cycle RISC-V CPU on the DE0-Nano board provided valuable insights into hardware design, optimization, and debugging. The CPU successfully executed instructions and produced the expected outputs, validating the correctness of our Verilog implementation.

Key learnings -

- Portability: We learned how to make Verilog code cross-compatible, ensuring smooth transfer from one development environment (Quartus Prime) to another (Vivado) while maintaining its logic. This required careful handling of syntax differences, tool-specific configurations, and resource constraints to ensure successful implementation.

- Understanding Essential Components in FPGA Development:

- Block Diagram: A visual representation of how different modules and IPs are interconnected, helping in organizing the design and debugging.

- Wrapper File: A top-level file that integrates all the submodules and ensures seamless communication between them.

- Constraints File: Defines the pin mappings, clock constraints, and timing requirements to ensure the design is correctly implemented on hardware.

- IP Sources: Pre-built Intellectual Property (IP) cores, such as memory controllers and communication interfaces, which can be integrated into the design to enhance functionality without needing to build everything from scratch.

Improvements or Modifications Needed

In the future, we plan to integrate actual RAM into the CPU, allowing instructions to be loaded externally rather than hardwired. While hardwiring is beneficial for optimizing specific tasks, as we have done here, adding flexibility would enhance adaptability for broader applications. Additionally, we aim to natively run TinyML commands, including MicroSpeech, which may require further optimizations to improve efficiency and compatibility.

Further improvements include refining the CPU’s pipeline structure to enhance instruction throughput and exploring multi-core integration for parallel processing. Power optimization techniques should also be considered, especially for resource-constrained applications like space rovers. Lastly, increasing compatibility with standard peripherals and communication protocols would expand the CPU’s usability across various embedded and AI-driven applications.