IIIT-B Chip Design Studio Weekly Report

Week: 4

Team Name/Project: Team Friday - RISC V Single Cycle CPU + More

Team Members:

Namai Chandra 23f3000200@es.study.iitm.ac.in

Madhur Thareja 23f3000178@es.study.iitm.ac.in

Aditya Prakash 23f3000345@es.study.iitm.ac.in

Vivek Dhariwal 23f3000338@es.study.iitm.ac.in

# 1. Updates

Current Progress:

The pipelined CPU is being upgraded with an additional Vector Processing Unit (VPU), which has the potential to evolve into a GPU in the future. The VPU is the first step towards performing calculations involving matrices, which further leads to implementing convolution and then several neural networks. Additionally, power efficiency has been a key focus, with optimizations enabling the CPU to operate entirely on ambient energy harvested from environmental radio frequencies. The Logic Equalance Test for the CPUs has been conducted to ensure that it is logically producing the desired output. The number of cycles has been counted. The circuit in simulation for energy harvesting has also been developed and is capable of powering it, with ballpark calculations evident, creating a seamless integration of digital and analog systems that perfectly embody the essence of IIIT-B.

Both the CPUs have been tested and implemented on FPGAs, the single cycle on De0 Nano and the pipelines on the Xilinx ZCU104. Both are capable of running the desired C path planner program while also being low powered.

Challenges Faced:

- Optimizing clock speed and power consumption is a tradeoff; increasing one affects the other. Hence optimization is the key that has been used. Pareto Optimality dictates that improving one metric necessitates sacrificing another. Our goal is to challenge this tradeoff.

- For the power harvesting analog component of our project, estimating the energy that could be harvested from ambient radio frequencies was challenging, as this could not be accurately simulated. The simulated circuit failed to provide accurate data because an antenna is required to capture RF waves.

- Despite the correct generation of the bitstream, the VIO and ILA outputs were not displayed after connecting the board and uploading the code. This issue was observed when the board was changed after synthesis.

- The ‘Run Automated Block Connection’ did not function correctly as it created unnecessary external outputs. Moreover, the testbench had to be properly imported, including the .dump file. The block connection was unnecessary since we followed the instantiation approach.

- With the introduction of the VPU, differentiating between scalar and vector instructions became a challenge. The existing ALU was unable to handle vector operations, so the ALU was separated based on scalar and vector values. Similarly, other modules were also structured to segregate these two types of values for more efficient processing.

- GitHub Link:

Repository: https://github.com/NamaiBest/iiitb_chip_design_studio

- Project Idea

This is in addition to what was mentioned in the week 1,2&3 project idea report

The following is the progress till now w.r.t the CPUs and their deployment:

| CPU | Implementation | Specifications | Instructions per cycle | Throughput |
| --- | --- | --- | --- | --- |
| Single Cycle | De0 Nano, PYNQ Z1 | CPI: 1 | 1 | 1 MIPS. |
| Pipelined | Xilinx ZCU-104 | CPI: 1.3 | 0.76* | 1 MIPS. |

The pipeline CPU is being upgraded with a VPU but it is yet to be integrated with the FPGA

The VPU (Vector Processing Unit) operates with a dedicated controller and ALU, which determines execution based on the is_scalar variable. If is_scalar is low, instructions are offloaded to the VPU for parallel processing; otherwise, the scalar ALU handles execution. The VPU architecture includes multiple specialized modules such as

vector_alu.v, vector_datapath.v, vector_muxes.v, vector_branching_unit.v, vector_decoder.v, vector_pipeline.v, vector_controller.v, vector_hazard_unit.v, and vector_reg_file.v.

These modules collectively enable efficient vectorized computations, optimizing performance for workloads involving parallel data processing.

We plan to harvest RF energy from frequencies ranging from 900 MHz to 3.5 GHz, primarily from mobile networks. The system begins with a custom-designed antenna, where impedance matching is implemented to ensure maximum power transfer. The harvested RF signal is then processed through a full-wave rectifier, which converts the AC signal into a usable DC voltage. Finally, a voltage doubler circuit, consisting of two capacitors and diodes, further boosts the rectified voltage to enhance energy storage efficiency.

When implemented using an ASIC flow and operated at 1MHz, the CPU achieves optimal power efficiency while executing all required instructions within the designated timeframe. This ensures that the system can be fully powered by the energy harvested from RF signals.

As for the single cycle, CPU it's GDS along with all of the necessary instructions had been hardwired and the final GTS file has been attached in the folder Week 2/Single Cycle CPU_team_friday_week2/Major Output Files that is ready for tapeout.

# 4. Schematic/Simulations:

Final Analysis for Single Cycle:

Single Cycle on ASIC

Instruction Size: 32-bit instructions

Word Size: 4 bits

Size of Instruction Memory: 512 bits /4 = 128

- DRC Pass

- GDS

Single Cycle on FPGA

Test Map

Test Statement:

Present Node = 8

Final Node = 23

Real Time Simulation Output

For the shortest path from 8 to 11, it will indeed encounter the nodes 6,0,1 then 11. This is done through the signal tap logic analyzer present in Quartus Prime.

The pathplanner.c is running successfully, the input of start point in the first 2 location of data memory and it calculates and stores the correct path in the DataMem.

Final Analysis for Pipelined:

Pipelined on ASIC

- Mapping Logic Gates:

- GUI Show:

- Logic Equivalence Verification:

Pipelined on FPGA

- Bitstream Generation:

- Design Schematics:

- Synthesis:

- Implemented Design:

- ILA Inputs Working on ZCU 104

5. Analysis:

Radio Frequency Energy Harvester

Single Cycle CPU: 2.44mW

Pipeline CPU: 3.4-6.8mW

Ballpark calculations for power estimates

Key Findings

Summary - This project focuses on developing a modular low pwer, RISC-V CPU on an FPGA and ASIC for efficient edge computing. The design follows a structured module instantiation approach, ensuring scalability and optimized performance. Key implementations include a pipelined and single cycle CPU, real-time path-planning algorithms, and FPGA-accelerated computations for autonomous navigation. The CPU has been successfully migrated from Quartus to Xilinx Vivado and deployed on the PYNQ-Z1/Z2, enabling Python-based interaction with hardware. To enable AI capabilities, a Vector Processing Unit (VPU) is being integrated for matrix operations, crucial for CNNs. Additionally, the system is designed for ultra-low power consumption, leveraging an RF energy harvesting setup for sustainable operation. Practical applications include offline ambulance navigation in remote areas and autonomous lunar/martian rover pathfinding. Future work involves integrating TinyML for lightweight AI processing and hardware acceleration while refining energy-efficient neural computations.

Final Implementations:

- Single Cycle:

ASIC:

- Analysis and Synthesis (Genus)

- GUI Map

- GDS (Innovus)

- LEC (Conformal)

FPGA:

- Implementation (Quartus Prime)

- RTL Simulation (Model Sim Altera)

- Implemented on De0 Nano FPGA

- Signal Tap Logic Analysis for verification

- Pipeline + VPU (Partially):

ASIC:

- Analysis and Synthesis (Genus)

- Elaboration

- GUI Map

FPGA:

- Implementation (Vivado)

- RTL Simulation (Vivado)

- Implementation (ZCU104)

- Signal Tap

Insights or Learnings

Key Learnings -

- Identified minor optimization techniques in the ALU that can enhance CPU performance slightly.

- The implementation of a Vector Processing Unit (VPU) necessitates designing a three-dimensional instruction memory to facilitate efficient vector processing.

- Deploying the single-cycle RISC-V CPU on the DE0-Nano board provided crucial insights into hardware design, performance tuning, and debugging methodologies. The successful execution of instructions confirmed the correctness of our Verilog implementation.

- Gained a deeper understanding of integrating multiple hardware modules, such as controllers and memory units, while ensuring seamless communication between them.

- For the CPU to receive instructions, either a separate Block RAM (BRAM) module can be created to load instructions externally, or the instructions can be hardwired for specific tasks, balancing flexibility and optimization.

Improvements or Modifications Needed

- Aim to natively run TinyML commands, including MicroSpeech, with further optimizations for enhanced efficiency and compatibility.

- We plan to refine the CPU’s pipeline architecture for full convolution support.

- Intend to implement RF harvesting by further fine-tuning the circuit.